| Theme | Dataset/file used | Provider or source | Spatial/temporal detail | Derived predictor(s) | citation |
| --- | --- | --- | --- | --- | --- |
| Study area | Administrative boundary_Oxfordshire.shp | UK boundary dataset or project boundary layer | Oxfordshire polygon boundary | Clipping mask and reporting unit | Office for National Statistics, n.d.; UK Government, n.d. |
| Elevation | United_Kingdom_Copernicus_GLO30_DEM_Mosaic.tif | Copernicus DEM GLO-30 | Approx. 30 m global DEM/DSM | Elevation, slope, curvature, flow accumulation, TWI | Copernicus Data Space Ecosystem, n.d.; ESA, 2021 |
| Flood inventory | Flood_Zones_2_3_Rivers__Clip_Project.shp | Environment Agency Flood Map for Planning | Flood Zones 2 and 3 for rivers and sea | Flood/non-flood training label | Environment Agency, n.d.-a |
| Historical flooding | UK_Historical_Flood_Extent_2000_2018.tif | Environment Agency / project-derived historical flood extent | Rasterised historic flood evidence | Flood training support or overlay validation | Environment Agency, n.d.-b |
| Hydrology | WatercourseLink.shp | Ordnance Survey Open Rivers or EA watercourse network | Mapped river and stream lines | Distance to river, drainage density | Ordnance Survey, n.d.-a; Environment Agency, n.d.-a |
| Roads | Roads.shp | Ordnance Survey Open Roads or equivalent project road layer | Road-line network | Distance to roads, road density | Ordnance Survey, n.d.-b |
| Buildings | Building.shp | Ordnance Survey OpenMap Local / project building footprints | Building polygons or footprints | Building density | Ordnance Survey, n.d.-d |
| Greenspace | GB_GreenspaceSite.shp | Ordnance Survey Open Greenspace | Public greenspace polygons | Distance to greenspace, greenspace density | Ordnance Survey, n.d.-c |
| Land cover | United_Kingdom_LULC_2025.tif | Copernicus / project-derived LULC product | Categorical land-cover raster | Land-cover class | Karra et al., 2021 |
| Superficial geology | UK_625k_SUPERFICIAL_Geology_Polygons.shp | British Geological Survey BGS Geology 625k | Regional geological polygons | Superficial geology class | British Geological Survey, n.d.-a |
| Bedrock geology | 625k_V5_BEDROCK_Geology_Polygons.shp | British Geological Survey BGS Geology 625k | Regional geological polygons | Bedrock geology class | British Geological Survey, n.d.-a; British Geological Survey, n.d.-b |
| Rainfall | UK_Mean_Annual_Rainfall_1950_2025_UK_Clipped.tif | ERA5-Land Daily Aggregated climate reanalysis, ECMWF / Copernicus Climate Change Service, accessed through Google Earth Engine dataset ECMWF/ERA5_LAND/DAILY_AGGR | Daily aggregated climate reanalysis, 1950–2025; approximately 11 km pixel size | Mean annual rainfall | ECMWF, n.d.; Copernicus Climate Change Service, n.d.; Google Earth Engine, n.d.; Muñoz-Sabater et al., 2021 |
| Rainfall extremes | UK_Mean_Annual_Max_Daily_Rainfall_1950_2025_UK_Clipped.tif | ERA5-Land Daily Aggregated climate reanalysis, ECMWF / Copernicus Climate Change Service, accessed through Google Earth Engine | Daily precipitation totals derived from the total_precipitation_sum band and converted from metres to millimetres | Annual maximum daily rainfall; extreme rainfall frequency | ECMWF, n.d.; Copernicus Climate Change Service, n.d.; Google Earth Engine, n.d.; Muñoz-Sabater et al., 2021 |
| Seasonal rainfall | UK_Mean_Winter/Autumn/Spring/Summer_Rainfall_*.tif | ERA5-Land Daily Aggregated climate reanalysis, ECMWF / Copernicus Climate Change Service, accessed through Google Earth Engine | Seasonal aggregation of daily precipitation totals for winter, spring, summer and autumn, 1950–2025 | Seasonal rainfall predictors | ECMWF, n.d.; Copernicus Climate Change Service, n.d.; Google Earth Engine, n.d.; Muñoz-Sabater et al., 2021 |
| Rainfall anomaly | UK_Rainfall_Anomaly_2025_UK_Clipped.tif | Project-derived anomaly from rainfall baseline | 2025 rainfall anomaly relative to the long-term 1950–2025 rainfall baseline | Rainfall anomaly predictor | ECMWF, n.d.; Copernicus Climate Change Service, n.d.; Google Earth Engine, n.d.; Muñoz-Sabater et al., 2021 |